ULOG3

Image Processing Team

Project Title:

Water Body Segmentation Using NIR Imagery

Team Members:

- Ranjitha – Team Leader

- Swedha Varshini C

- Mohammed Riyan H

Abstract:

This project aims to develop an automated system for segmenting water and land areas from Near-Infrared (NIR) satellite images. By utilizing the distinct reflectance properties of NIR, where water bodies absorb and land surfaces reflect, the system generates precise segmentation masks. The solution enhances large-scale environmental monitoring, enabling accurate water resource management and reducing manual effort through advanced image processing and machine learning techniques.

Objective:

The objective of this project is to design and implement an automated system that accurately detects and quantifies water and land areas from Near-Infrared (NIR) satellite images. By leveraging the unique reflectance properties of NIR, the system will generate precise segmentation masks, enabling scalable environmental monitoring, efficient water resource management, and minimizing manual intervention in geographic analysis.

Vision:

To create an intelligent and automated system that enables accurate and large-scale monitoring of Earth’s water and land resources using satellite imagery, providing reliable geographic insights to support sustainable development, environmental management, and data-driven decision-making through advanced image processing and machine learning technologies.

Mission:

To develop an automated NIR image segmentation system that accurately distinguishes water and land areas, utilizing deep learning techniques to deliver precise, scalable, and efficient solutions for environmental monitoring and resource management.

1. Introduction:

- Satellite imagery has become an essential tool for observing and analysing Earth's surface, enabling applications in environmental monitoring, urban planning, disaster management, and resource assessment. Among various imaging technologies, Near-Infrared (NIR) imaging plays a crucial role in differentiating between water bodies and land surfaces due to its unique interaction with moisture and vegetation.

- The NIR spectrum, ranging from 700nm to 1100nm, is invisible to the human eye but is highly effective in remote sensing. Water bodies exhibit high absorption of NIR wavelengths, making them appear dark in NIR images, whereas land surfaces, especially vegetation, reflect NIR strongly and appear bright. These distinct reflectance properties allow for clear separation between water and land regions, which is often challenging in visible spectrum images due to atmospheric conditions, turbidity, or varying surface textures.

- Manual interpretation and segmentation of satellite images are time-consuming, prone to errors, and not scalable for large datasets. The need for an automated, accurate, and efficient system to classify water and land areas has become critical, especially for applications involving real-time environmental monitoring and resource management.

- This project focuses on leveraging the physics of NIR reflectance in combination with deep learning-based image processing algorithms to develop a robust and automated segmentation system. By generating precise segmentation masks that differentiate between water bodies and land surfaces, the system aims to facilitate large-scale analysis with minimal manual intervention, ensuring high accuracy across diverse terrains and environmental conditions.

2. Physics Of NIR Reflectance:

2.1 What Is NIR:

NIR stands for Near Infrared, which refers to a segment of the electromagnetic spectrum with wavelengths from approximately 750 nm to 2500 nm. It is commonly used in Near-Infrared Spectroscopy (NIRS), an analytical technique that utilizes near-infrared radiation to analyze the chemical and physical properties of samples, both in solids and liquids. NIR is invisible to the human eye and is positioned just beyond the visible red.

Fig 1. Electromagnetic Spectrum

2.2 Why NIR:

- Non-destructive sensing – Doesn't damage materials.

- High penetration ability – Passes through atmospheric particles like haze, smoke, and clouds better than visible light.

- Thermal independence – Unaffected by temperature variations during daytime or nighttime.

- High contrast between materials – Especially between water and land surfaces due to their differing absorption and reflection rates.

2.3 How NIR Camera Works:

NIR (Near-Infrared) cameras work by capturing the reflected or emitted radiation from objects in the near-infrared spectrum (typically 700–1100 nm). To ensure only NIR wavelengths are observed from full-spectrum sunlight, the camera uses gratings (to split light by wavelength) and polarizers (to filter specific orientations of light waves). This helps isolate the desired NIR band and eliminate unwanted visible light or noise.

Fig 2. Working of Polarizers

When NIR light interacts with materials on Earth’s surface, four primary behaviours can occur:

- Absorption – The material absorbs the NIR light and converts it to internal energy.

- Emission – The material emits radiation, including NIR, depending on its temperature or properties.

- Absorption + Emission – Some materials first absorb and then re-emit part of the energy.

- Reflection – The surface reflects the incoming NIR radiation without absorption.

Fig 3. Photoelectric Effect

2.4 Grayscale Representation

Each pixel in an NIR image corresponds to a brightness value from 0 to 255:

- 0 (Pure Black) → Indicates very low reflection or emission, meaning most of the NIR was absorbed (e.g., water).

- 255 (Pure White) → Indicates high reflection or emission, meaning the surface strongly reflects NIR (e.g., dry soil, land, vegetation).

This physical interaction is the core principle behind distinguishing materials like water and land in NIR images:

- Water absorbs NIR heavily → reflects less → appears darker.

- Land reflects more NIR → appears brighter.

This difference in pixel brightness forms the basis for image segmentation and classification in NIR-based Earth observation.

Fig 4. Pixel Intensity Representation

2.5 Mathematical Representation:

When Near-Infrared (NIR) radiation interacts with a surface, the total incident energy Ein distributed into three components:

Ein = Eabs + Eref + Eemi

Where:

Ein - Incident NIR energy

Eref - Energy Reflected

Eabs - Energy Absorbed by material

Eemi - Energy Emitted

For Earth surface materials in NIR satellite imaging, we usually assume no significant emission (especially during the day), so:

Ein ≈ Eabs + Eref

Reflectance Coefficient R:

R= ErefEin

Where:

- R ∈ [0,1]

- High Ref → High reflection (e.g., land/vegetation)

- Low Ref→ Low reflection (e.g., water)

In image processing, this reflectance R is mapped to pixel intensity I ∈ [0,255]

Ipx = 255 x R

Thus:

R ≈ 0  I ≈ 0  Black pixel (low reflectance, water)

R ≈ 1  I ≈ 255  White pixel (high reflectance, land)

Case Study: Interaction of NIR Light with Different Surfaces

| Surface Type | NIR Reflectance (%) | Pixel Value (0–255) | Appearance |
| --- | --- | --- | --- |
| Water | 5% | ~13 | Very Dark |
| Bare Soil | 30–40% | ~76–102 | Mid-gray |
| Vegetation | 50–60% | ~127–153 | Bright |
| Snow/Ice | 80–90% | ~204–230 | Very Bright |

a) Case 1: Water Surface

- Input NIR Light: 100%

- Absorption: 60%

- Reflection/Emission: 40%

- Behaviour:

  - Water absorbs a significant portion of the incoming NIR radiation due to its molecular structure (hydrogen bonds absorb longer wavelengths).

  - Very little light is reflected back to the sensor.

- Pixel Value:

Ipx = R × 255 =0.4×255=102

Appears mid-Black in the image.

b) Case 2: Land Surface

- Input NIR Light: 100%

- Absorption: 40%

- Reflection/Emission: 60%

- Behaviour:

  - Soil and vegetation reflect a large portion of NIR.

  - Especially healthy vegetation reflects more NIR due to internal leaf structures (spongy mesophyll).

- Pixel Value:

Ipx = R x 255 = 0.6 x 255 = 153

Appears grey in the image.

Fig 5. Comparison of RGB and NIR Views

2.6 Physics Final Word

Ipx ∝ ReflectanceAbsorption

2.7 Impact on CNN-based Water-Land Segmentation

- This consistent contrast is exploited by Convolutional Neural Networks (CNNs) and segmentation models.

- The model learns from these physical differences during training:

  - Lower pixel regions = Water

  - Higher pixel regions = Land

3. Methodology

3.1 Dataset Collection

- The dataset consists of Near-Infrared (NIR) satellite images sourced from Google Cloud Storage.

- Images are in TIFF (.tif) format, preserving high-resolution spectral data for accurate segmentation.

- Each image has a spatial resolution of 516 × 513 pixels.

- The dataset includes various geographic locations and environmental conditions for better generalization.

- The dataset is split into 70% for training, 15% for validation, and 15% for testing.

- This split ensures balanced and effective training, fine-tuning, and evaluation.

3.1.1 Summary:

| Dataset Partition | Number of Images | Percentage |
| --- | --- | --- |
| Training Set | X images | 70% |
| Validation Set | Y images | 15% |
| Test Set | Z images | 15% |

3.2 Data Preprocessing

To ensure consistent input to the segmentation model and enhance training efficiency, a comprehensive preprocessing pipeline was implemented for the Near-Infrared (NIR) satellite images. The key steps include:

a) Recursive Folder Scanning & File Handling

- A custom data generator was designed to recursively scan the dataset directory, identifying all relevant image files in formats such as TIFF (.tif/.tiff), JPG, JPEG, and PNG.

- This dynamic and flexible approach ensures compatibility with varied dataset organizations.

b) NIR Band Extraction & Normalization

- For multi-band TIFF images, the NIR band (usually the last channel) is extracted.

- The extracted data is normalized to a 0–255-pixel intensity range using OpenCV’s normalization functions.

- This normalization ensures uniform pixel values, which is critical for stable and consistent model training.

c) Image Resizing

All images are resized to 256×256 pixels, maintaining a balance between:

- Retaining spatial detail

- Reducing computational cost

This ensures a uniform input shape to the CNN model.

d) On-the-Fly Binary Mask Generation (Otsu Thresholding)

Instead of relying on manually labelled masks, a dynamic mask generation strategy using Otsu’s thresholding is employed. For each input image:

- Water regions (high NIR absorption) → Foreground (1)

- Land regions (high NIR reflectance) → Background (0)

This real-time mask generation provides a robust and automated labelling mechanism, ideal for scaling the segmentation pipeline.

e) Data Augmentation (Optional for Future Work)

Although basic normalization and resizing are implemented, data augmentation (e.g., flipping, rotation, scaling) is identified as a potential enhancement to:

- Improve model generalization

- Reduce overfitting risks

This can be incorporated in future iterations of the pipeline.

3.3 CNN Model Architecture

For the task of water-land segmentation, a Convolutional Neural Network (CNN) based encoder-decoder architecture was designed, optimized for binary segmentation of NIR satellite images.

a) Encoder (Down sampling Path)

The encoder extracts hierarchical spatial features using a series of convolutional and pooling layers:

- Conv2D layers with 32, 64, and 128 filters, using 3×3 kernels, activated by ReLU, to learn spatial patterns.

- Batch Normalization is applied after each convolution to stabilize training and speed up convergence.

- MaxPooling2D layers down sample feature maps by a factor of 2, enabling the model to learn spatial hierarchies and contextual features.

b) Decoder (Up sampling Path)

The decoder reconstructs the feature maps to match the original input resolution, generating the segmentation mask:

- UpSampling2D layers increase spatial resolution step-by-step.

- Each up sampling is followed by Conv2D layers (with 64 and 32 filters) and ReLU activation, to refine features.

- Batch Normalization ensures model stability during reconstruction.

c) Output Layer

The final layer is a 1×1 Conv2D layer with a sigmoid activation function, which outputs a binary mask:

- Each pixel represents the probability of being Water (foreground = 1) or Land (background = 0).

| Layer Type | Details |
| --- | --- |
| Input Layer | 256x256x1 grayscale NIR image |
| Conv2D + Batch Norm | 32 filters, 3x3 kernel |
| MaxPooling2D | 2x2 pool size |
| Conv2D + Batch Norm | 64 filters, 3x3 kernel |
| MaxPooling2D | 2x2 pool size |
| Conv2D + Batch Norm | 128 filters, 3x3 kernel |
| UpSampling2D | 2x2 up sample |
| Conv2D + Batch Norm | 64 filters, 3x3 kernel |
| UpSampling2D | 2x2 up sample |
| Conv2D + Batch Norm | 32 filters, 3x3 kernel |
| Output Layer | 1 filter, 1x1 kernel, Sigmoid Activation |

3.4 Training Strategy

To ensure efficient learning and accurate segmentation, a well-structured training strategy was implemented. This strategy involved:

- Selecting an appropriate loss function — Binary Cross-Entropy (BCE), ideal for pixel-wise binary classification

- Using the Adam optimizer, known for adaptive learning rates and fast convergence

- Integrating callbacks such as Early Stopping and ReduceLROnPlateau to:

  - Prevent overfitting

  - Stabilize training

  - Optimize learning dynamics

This combined setup enabled the model to generalize effectively, minimize training loss, and maintain robust performance across unseen satellite images.

a) Loss Function

- The task of water-land segmentation is formulated as a binary classification problem at the pixel level. Therefore, Binary Cross-Entropy (BCE) Loss was used as the loss function.

- BCE is well-suited for segmentation tasks where the output mask consists of two classes:

  - Water → Foreground = 1

  - Land → Background = 0

- This loss function penalizes incorrect pixel classifications, effectively guiding the model toward accurate mask generation by minimizing the difference between predicted and true class probabilities at each pixel.

b) Optimizer

- The model was trained using the Adam Optimizer, an adaptive learning rate optimization algorithm known for its efficiency and faster convergence in deep learning tasks.

- Adam combines the advantages of both AdaGrad (adaptive learning for sparse data) and RMSProp (adaptive learning for non-stationary objectives), making it particularly effective for handling sparse gradients and non-stationary loss landscapes — common in segmentation problems.

c) Training Parameters

| Parameter | Value |
| --- | --- |
| Batch Size | 8 |
| Epochs | 10 |
| Learning Rate | Adaptive (Reduced on Plateau) |
| Input Image Size | 256 x 256 x 1 (Grayscale NIR) |

d) Callbacks to Enhance Training

To prevent overfitting and ensure optimal model convergence, the following callbacks were integrated into the training loop:

- Early Stopping:
Monitors the loss and halts training if it does not improve after 10 consecutive epochs. Automatically restores the best model weights, preventing performance degradation.

- ReduceLROnPlateau:
Monitors training stagnation and reduces the learning rate by a factor of 0.5 if the loss plateaus for 5 epochs. This enables finer weight updates during flat regions of the training curve.

e) Training Execution

The model was trained on the pre-processed dataset using the custom data generator, ensuring efficient batch loading and on-the-fly mask generation. The training was performed for 10 epochs, with callbacks actively managing learning dynamics.

3.5 Post-processing & Output Generation

- After the CNN model generates the initial segmentation masks, a series of post-processing steps are applied to refine the results and extract meaningful insights, such as area percentages of water and land regions.

- These steps ensure that the outputs are not just binary masks, but transformed into actionable geographic intelligence — suitable for applications in environmental analysis, GIS systems, and real-time decision-making.

a) Binary Mask Thresholding

The model outputs probability maps (values between 0 and 1) for each pixel. A fixed threshold (commonly 0.5) is applied to convert these probability maps into binary segmentation masks, where:

- Pixel value ≥ 0.5 → Classified as Water (Foreground = 1)

- Pixel value < 0.5 → Classified as Land (Background = 0)

b) Morphological Operations (Optional Enhancement)

To remove small noise and refine segmentation boundaries, morphological operations such as:

- Opening (Erosion followed by Dilation): Removes small false positives.

- Closing (Dilation followed by erosion): Fills small holes in water regions.
These steps enhance mask clarity, especially in noisy or complex terrains.

c) Area Percentage Calculation

- Post segmentation, the total pixel count for water and land regions is computed to determine the percentage area coverage:

- This quantification enables objective assessment of water body coverage in each satellite image.

d) Visualization of Results

The processed outputs are visualized through:

- Overlayed Segmentation Masks on Original NIR Images (for qualitative inspection)

- Histograms of Pixel Distributions (showing intensity spread)

- Pie Charts depicting the calculated Water vs Land area percentages for clear representation of segmentation results.

Test 01

Mask Prediction

Information From NIR Image

Test 02

Mask Prediction

Information From NIR Image

Test 03

Mask Prediction

Information From NIR Image

Test 04

Mask Prediction

Information From NIR Image

Test 05

Mask Prediction

Information From NIR Image

e) Saving Segmented Images

The final segmented masks, along with computed area percentages, are saved for each image. This output can be used for downstream applications such as environmental monitoring reports, GIS analysis, and real-time geographic intelligence systems.

4. Challenges Faced:

| Challenge | Explanation |
| --- | --- |
| Limited Availability of NIR Datasets | Publicly available NIR datasets specific to water body segmentation are scarce, requiring extensive searching and filtering to find suitable imagery. |
| Dataset Sourcing & Band Selection | The final dataset had to be carefully sourced from Google Cloud, ensuring NIR bands were present and usable for segmentation tasks. |
| Environment & Library Compatibility | Setting up the development environment with TensorFlow, OpenCV, scikit-image, and tifffile caused compatibility issues across different systems. |
| Dependency Troubleshooting | Resolving library dependencies and configurations consumed significant time, requiring manual interventions and troubleshooting. |
| Corrupted Image Preprocessing | Several images became corrupted (fully black) during preprocessing due to format inconsistencies, necessitating dataset reloading and careful file handling. |
| Data Splitting Complexity | The dataset was originally unstructured (single folder), requiring custom scripts to automate the split into Train (70%), Validation (15%), and Test (15%). |
| Computational Limitations | Processing high-resolution NIR images and training segmentation models demanded substantial computational resources, leading to memory optimization challenges. |

5. Conclusion

This project presents an automated system for segmenting and quantifying water and land areas from NIR satellite images using a CNN-based approach. By harnessing the physics of NIR reflectance and dynamic mask generation, the model achieves accurate, scalable, and efficient segmentation. The solution supports real-time environmental monitoring, reduces manual efforts, and lays the foundation for intelligent geographic analysis in sustainable resource management.